#include <WiFiS3.h>

#include <ThreeWire.h>

#include <RtcDS1302.h>

/* ---------- WIFI ---------- */

char ssid[] = "iPhone";

char pass[] = "12345678900";

WiFiClient client;

const char* server = "api.thingspeak.com";

String apiKey = "F93N81ZV09W85NI8";

unsigned long lastSend = 0;

const unsigned long interval = 15000;  // ThingSpeak limit

/* ---------- PIN DEFINITIONS ---------- */

#define TRIG_PIN       3

#define ECHO_PIN       4

#define TILT_PIN       5

#define RTC_CLK        6

#define RTC_DAT        7

#define RTC_RST        8

#define GREEN_LED      9

#define YELLOW_LED     10

#define RED_LED        11

#define VIBRATION_PIN  A0

/* ---------- RTC ---------- */

ThreeWire myWire(RTC_DAT, RTC_CLK, RTC_RST);

RtcDS1302<ThreeWire> rtc(myWire);

/* ---------- SECURITY STATES ---------- */

enum SecurityState {

SAFE = 5,

SUSPICIOUS = 1,

INTRUSION = 2

};

void setup() {

Serial.begin(9600);

while (!Serial);

/* ---------- PIN MODES ---------- */

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(TILT_PIN, INPUT_PULLUP);

pinMode(VIBRATION_PIN, INPUT);

pinMode(GREEN_LED, OUTPUT);

pinMode(YELLOW_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

/* ---------- RTC ---------- */

rtc.Begin();

if (!rtc.GetIsRunning()) rtc.SetIsRunning(true);

if (!rtc.IsDateTimeValid()) rtc.SetDateTime(RtcDateTime(__DATE__, __TIME__));

/* ---------- WIFI ---------- */

Serial.print("Connecting to WiFi");

while (WiFi.begin(ssid, pass) != WL_CONNECTED) {

delay(5000);

Serial.print(".");

}

Serial.println("\nWiFi connected");

Serial.println("Smart Home Intrusion System Started");

}

void loop() {

/* ---------- ULTRASONIC ---------- */

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000);

int distance = (duration == 0) ? -1 : duration * 0.034 / 2;

/* ---------- VIBRATION ---------- */

int vibration = analogRead(VIBRATION_PIN);

/* ---------- TILT ---------- */

bool tilt = digitalRead(TILT_PIN) == LOW;

/* ---------- RTC ---------- */

RtcDateTime now = rtc.GetDateTime();

bool night = (now.Hour() >= 22 || now.Hour() <= 6);

/* ---------- SECURITY LOGIC ---------- */

SecurityState state;

if (distance > 0 && distance < 100 && night) {

state = INTRUSION;

}

else if (tilt || vibration > 600) {

state = SUSPICIOUS;

}

else {

state = SAFE;

}

/* ---------- LED OUTPUT ---------- */

digitalWrite(GREEN_LED,  state == SAFE);

digitalWrite(YELLOW_LED, state == SUSPICIOUS);

digitalWrite(RED_LED,    state == INTRUSION);

/* ---------- SERIAL MONITOR ---------- */

Serial.println("---------- STATUS ----------");

Serial.print("Time       : ");

Serial.print(now.Hour()); Serial.print(":");

Serial.print(now.Minute()); Serial.print(":");

Serial.println(now.Second());

Serial.print("Distance   : "); Serial.println(distance);

Serial.print("Vibration  : "); Serial.println(vibration);

Serial.print("Tilt       : "); Serial.println(tilt ? "YES" : "NO");

Serial.print("Night      : "); Serial.println(night ? "YES" : "NO");

Serial.print("State      : ");

if (state == SAFE) Serial.println("SAFE");

else if (state == SUSPICIOUS) Serial.println("SUSPICIOUS");

else Serial.println("INTRUSION");

Serial.println("----------------------------");

/* ---------- THINGSPEAK SEND ---------- */

if (millis() - lastSend > interval && WiFi.status() == WL_CONNECTED) {

if (client.connect(server, 80)) {

String url = "/update?api_key=" + apiKey +

"&field1=" + String(distance) +

"&field2=" + String(vibration) +

"&field3=" + String(state);

client.print(String("GET ") + url + " HTTP/1.1\r\n" +

"Host: " + server + "\r\n" +

"Connection: close\r\n\r\n");

Serial.println("Data sent to ThingSpeak");

} else {

Serial.println("ThingSpeak connection failed");

}

client.stop();

lastSend = millis();

}

delay(2000);

}